IIO Technologies

Signal Generator

API Integration Guide

| Version | 1.0.0 |
| --- | --- |
| Product | IIO Signal Generator |
| API Base URL | http://localhost:3333 |
| Format | REST / JSON over HTTP |
| Transport | RS422 Serial — CRC-16 Modbus |

# 1.  REST API Reference

Base URL: http://localhost:3333   |   All request/response bodies are JSON   |   Available in Server Mode and UI Mode simultaneously.

## Endpoint Overview

| Method | Endpoint | Body | Description |
| --- | --- | --- | --- |
| GET | /ports | — | List all available COM/serial ports |
| POST | /connect | { port, baudRate? } | Connect to the SDR device |
| POST | /disconnect | — | Disconnect from the device |
| GET | /status | — | Full status: connection + all 4 modules |
| GET | /status/:moduleId | — | Status for a single module (1–4) |
| POST | /configure | { moduleId, jammingCode?, frequency?, ch1Gain?, ch2Gain? } | Apply jamming code + frequency + gain |
| POST | /jamming/start | { moduleId } | Start jamming on a specific module |
| POST | /jamming/stop | — | Stop jamming on ALL modules (broadcast) |

## GET  /ports

Returns a list of available serial ports on the system.

### Response

{ "success": true, "ports": [ { "path": "COM3", "manufacturer": "IIO Technologies" }, { "path": "COM6", "manufacturer": "..." } ] }

## POST  /connect

Opens a serial connection to the SDR device. After successful connection, all 4 modules are automatically initialised (status, jamming list, frequency range fetched).

### Request Body

{ "port": "COM3", // required — COM port path "baudRate": 115200 // optional — default 115200 }

### Response

// Success { "success": true, "message": "Connected to COM3" } // Failure { "success": false, "error": "Port not found" }

## POST  /disconnect

Closes the active serial connection and resets all module states.

### Response

{ "success": true }

## GET  /status

Returns the full system state including connection info and all 4 module states.

### Response

{ "success": true, "connectionStatus": "CONNECTED", "connectedPort": "COM3", "modules": { "1": { "jammingActive": false, "activeJammingCode": 11, "frequency": 885.0, "deviceData": { "temperature": 42, "voltage": 12.05, "alarms": { "iAlarm": false, "tAlarm": false, "antAlarm": false }, "adcA": 1.24, "adcB": 1.22 }, "freqLimits": { "ch1": { "lower": 800000, "upper": 1000000, "current": 885000 } } }, "2": { ... }, "3": { ... }, "4": { ... } } }

## GET  /status/:moduleId

Returns the state of a single module. moduleId must be 1, 2, 3, or 4.

### Example

GET http://localhost:3333/status/1

### Response

{ "success": true, "moduleId": 1, "jammingActive": false, "activeJammingCode": 11, "frequency": 885.0, "deviceData": { "temperature": 42, "voltage": 12.05, ... } }

## POST  /configure

Applies configuration to a module. Sends jamming code, then (optionally) frequency, then CH1/CH2 gain. Commands are sent with hardware-safe 300ms delays between each step.

### Request Body

{ "moduleId": 1, // required — target module (1–4) "jammingCode": 11, // optional — decimal value (see Jamming Code table); uses current if omitted "frequency": 885.0, // optional — MHz; skipped if not provided "ch1Gain": 23, // optional — integer 1–35; default 35 "ch2Gain": 23 // optional — integer 1–35; default 35 }

### Validation Rules

| Field | Rule | Error if invalid |
| --- | --- | --- |
| moduleId | Required. Must be 1, 2, 3, or 4 | 400 — moduleId must be 1, 2, 3, or 4 |
| jammingCode | Optional. Must be a valid code from the Jamming Code table | 400 — Invalid jammingCode: X. Valid values: 0, 1, 2... |
| frequency | Optional. Any positive number in MHz | Silently skipped if 0, null, or omitted |
| ch1Gain | Optional. Integer between 1 and 35 | 400 — ch1Gain must be an integer between 1 and 35 |
| ch2Gain | Optional. Integer between 1 and 35 | 400 — ch2Gain must be an integer between 1 and 35 |

### Response

{ "success": true, "message": "Config sent to Module 1", "jammingCode": "OFDM_10M", "frequency": "885 MHz", "ch1Gain": 23, "ch2Gain": 23 }

### Error Response Example

// Invalid jamming code { "success": false, "error": "Invalid jammingCode: 99. Valid values: 0, 1, 2, 3..." } // Invalid gain { "success": false, "error": "ch1Gain must be an integer between 1 and 35" }

## POST  /jamming/start

Starts CH1 jamming on the specified module (0x02 = CH1 on, CH2 off). The module must be connected and configured first.

### Request Body

{ "moduleId": 1 // required — target module (1–4) }

### Response

{ "success": true, "message": "Jamming started on Module 1" }

## POST  /jamming/stop

Broadcasts a stop command to ALL modules simultaneously (module byte = 0x00). No body required.

### Response

{ "success": true, "message": "Jamming stopped on all modules" }

# 2.  Jamming Code Reference

Use the numeric value (decimal) in the jammingCode field of /configure.

| Code | Name | Code | Name | Code | Name |
| --- | --- | --- | --- | --- | --- |
| 0x00 (0) | CW | 0x0A (10) | OFDM_5M | 0x14 (20) | LFM_20M |
| 0x01 (1) | TBS_868 | 0x0B (11) | OFDM_10M | 0x15 (21) | LFM_70M |
| 0x02 (2) | TBS_915 | 0x0C (12) | OFDM_20M | 0x16 (22) | LFM_100M |
| 0x03 (3) | TBS_868+915 | 0x0D (13) | OFDM_70M | 0x17 (23) | LFM_150M |
| 0x04 (4) | ELRS_868 | 0x0E (14) | OFDM_100M | 0x18 (24) | LFM_140M |
| 0x05 (5) | ELRS_915 | 0x0F (15) | OFDM_150M | 0x19 (25) | LFM_200M |
| 0x06 (6) | ELRS_2450_A | 0x10 (16) | OFDM_140M | 0x1A (26) | OFDM_80M |
| 0x07 (7) | ELRS_868+915 | 0x11 (17) | OFDM_200M | 0x1D (29) | TBS+ELRS_30M |
| 0x08 (8) | TBS+ELRS_A | 0x12 (18) | LFM_5M | 0x21 (33) | TBS+ELRS_60M |
| 0x09 (9) | GNSS_70M | 0x13 (19) | LFM_10M |  |  |

# 3.  Usage Examples  (curl)

## 1.  List Ports

curl http://localhost:3333/ports

## 2.  Connect to Device

curl -X POST http://localhost:3333/connect \ -H "Content-Type: application/json" \ -d "{\"port\":\"COM3\"}"

## 3.  Configure Module 1  (OFDM_10M @ 885 MHz, Gain 23)

curl -X POST http://localhost:3333/configure \ -H "Content-Type: application/json" \ -d "{\"moduleId\":1, \"jammingCode\":11, \"frequency\":885, \"ch1Gain\":23, \"ch2Gain\":23}"

## 4.  Start Jamming on Module 1

curl -X POST http://localhost:3333/jamming/start \ -H "Content-Type: application/json" \ -d "{\"moduleId\":1}"

## 5.  Stop All Jamming

curl -X POST http://localhost:3333/jamming/stop

## 6.  Check Status

# All modules curl http://localhost:3333/status # Module 2 only curl http://localhost:3333/status/2

## 7.  Disconnect

curl -X POST http://localhost:3333/disconnect